Setup utility for the Psion Organiser II LZ

Copyright 1992-1997 Zac Schroff, all rights reserved.

--- License statement ---

This program is free for use and distribution as long as it is

distributed only in an unmodified state, this notice is left intact,

and the associated files are included with it :

SETUP.DOC      (this documentation file)

SETUP.OPL      (the source file)

SETUP.OB3      (the object only file)

If you intend to distribute modified of translated versions of this

program, please contact the author for permission.

You may modify the program to fit your needs as long as you do not

distribute modified versions of the program except as explicitly

provided for in the first paragraph of this section.

Neither the author of the program nor anybody who distributes it takes

any responsibility for how it will behave under the way you use or

abuse it on your system, nor any responsibility for damages to data,

other software, or hardware.  The limit of the liability of the author

or distributor is the sum which you paid for the program.

--- Software statement ---

This program is freeware.  You may use it as you see fit, as long as

you comply with the license statement above.

If you feel you'd like to send me something for this little program,

or if you just want to chat, or ask about using the program for your

own purpose, please contact me :

zschroff@docsware.com

Zac Schroff

2906 Firethorn Drive

Tuscaloosa, Alabama  35405

USA

--- Installation ---

Copy SETUP.OPL to your Organiser II LZ or LZ64.  Translate SETUP.OPL.

Save only the object to a RAMpack (to save space).  Install it in your

menu, or you can call it from within another program.

--- Operation ---

Run the SETUP.OPL object.  It will allow you to change certain key

settings on your organiser.  Use the LEFT and RIGHT arrow keys to

select which setting to view/change.  Use the UP and DOWN arrow keys

to change the setting.  Use S to save the settings to SYSTMCFG.ODB.

Use R to restore the settings from SYSTMCFG.ODB.  ON will exit the

program.

The settings should all be fairly obvious.  However, I will include

the settings names, a longer description, the default values, and any

warnings here...

Disp scroll dly horz

Display horizontal scrolling delay.  This is the number

of tics (1/20 second intervals) between shifts when a

line is being scrolled horizontally (as a calculator or

alarm or diary entry that is too long would be).  I would

recommend making this just fast enough that you can read

it comfortably.  Default value = 05

Disp scroll dly vert

Display vertical scrolling delay.  This is the number of

tics between shifts when the display is being scrolled

vertically (as when an OPL program tries to print several

lines of information).  I find most programs tend to avoid

scrolling, so this probably should be left alone.  Default

value = 14

Disp two-line border

Display two-line mode border fill character.  This is the

character that is used to fill around the display when you

use CM and XP translated modules.  Default value = 20

Key-board int divide

Key-board controller interrupt divide.  This is timebase for

the generation of the keyboard scan tic (used by several

other items, and called `tics' here).  WARNING : This should

not be changed unless you have a *VERY* good reason.

WARNING : A value of 0000 will cause the machine to hang.

Default value = B39F

Key-board rept delay

Key-board repeat delay.  This is the length of time in tics

that a key must be held down before it starts to repeat.

WARNING : Setting this to zero makes for very difficult

keyboard usage.  Default value = 05

Key-board rept div

Key-board repeat divider.  This is the length of time in

tics between keyboard repeats.  Default value = 01

Key-board power save

Key-board polling power save mode enable.  This is nonzero

to indicate that all unneeded devices are switched off when

any keyboard polling requests occur.  The documentation for

this seems intentionally vague...  Default value = FF

Key-board click len

Key-board click length.  This is the length of the key-board

click when a key is pressed or auto-repeats.  Psion alleges

this is measured in milliseconds, but I sort of think it's

measured in how many times it toggles the speaker.  NOTE :

if this is set to 01, most machines will make the key-click

even with sound disabled, but if set to any other value, the

sound disable affects the key-click.  WARNING : This can be

very annoying to your neighbours if you set it to a value

other than zero, one or two.  Default value = 01

Key-board caps lock

Key-board caps lock key scan code.  This is the scan code

for the key that (when used with shift) will toggle the caps

lock function.  NOTE : 00 disables caps lock toggling from

the key-board.  Default value = 20

Key-board nums lock

Key-board nums lock key scan code.  This is the scan code

for the key that (when used with shift) will toggle the nums

lock function.  NOTE : 00 disables nums lock toggling from

the key-board.  Default value = 21

Key-board shift flag

Key-board shift key disable.  This is nonzero to disable the

functioning of the shift key.  Note that when shift is

disabled, pressing it will function as a question mark key.

Default value = 00

Key-board spcl flags

Key-board special keys flags.  This badly documented value

indicates which special key combinations are enabled at any

particular time.  Apparently, it affects such obvious key

combinations as SHIFT-SPACE and SHIFT-EXE.  I can't find any

information on what each bit controls.  Default value = 00

Time work-day flags

Time work-day flags.  This indicates which day of the week

is a work-day.  Note bit seven is unused.  Other bits are

bit 6 = Sunday, bit 5 = Saturday, bit 4 = Friday, bit 3 =

Thudsday, bit 2 = Wednesday, bit 1 = Tuesday, bit 0 =

Monday.  Very useful if you work a nonstandard week.

Default value = 1F

Time DST(A),24hr(B)

Time daylight savings time and 24-hour mode flags.  This

indicates whether these options are set or not.  In this

case, the A and B functions of the program are useful, with

A toggling the DST flag, and B toggling the 24 hour flag.

Default value = 00

Time alarm chk flag

Time pending alarms check enable flag.  This is nonzero to

indicate the system will check alarms and sound them at the

proper time.  If this is zero, all alarms will be disabled

until this is set to a nonzero value.  Default value = 01

Time alarm check now

Time pending alarms check now flag.  This is nonzero to

indicate an alarm check is being made.  One can force an

alarm check at any time by setting this to a nonzero value,

and the next timer tic will check for pending alarms.  This

will always return to 00 after the alarm check is finished.

Default value = 00

Buzzer disable flag

Buzzer disable flag.  This is nonzero to disable sound

output from the Psion (see the note in the description for

the Key-board click len option, however).  To enable sound

again, set it to zero.  Default value = 00

Auto-off delay (sec)

Automatic switch off delay in seconds.  This is the amount

of time the Psion will wait (polling or waiting for a

keypress -- the Psion will never shut down until it is asked

about the keyboard!) after the last keypress before

switching itself off, in SECONDS.  Default value = 012C

Auto-off enable flag

Automatic switch off enable flag.  This is nonzero to

indicate the automatic switch off function is allowed to

turn the Psion off after the period indicated by the

Auto-off delay (sec) setting.  Default value = 01

Capitalise menu flag

Capitalise menu flag.  Actually, this should be renamed to

the display menu literally flag.  It is possible to embed

lower case letters in menus and they will still be displayed

in lower case.  Setting this to a nonzero value displays the

menus literally.  Setting this to zero will force the first

letter of each menu item to be capitalised and the rest of

the item to be lower case.  Default value = 00

Language running

Current language running on this machine.  This is Psion's

country code for the currently active language.  It appears

to be ignored except for the English, French, and German

values.  WARNING : changing this will probably affect the

language for the built-in submenus, and top-level entries

will still be valid but new ones will have to be added in

the new language.  Default value = (set by user on initial

battery insert)

Lang lock(A),Boot(B)

Language lock and second boot enable flags.  There does not

seem to be any documentation about these flags in any source

that is available to me.  Default value = 00

--- End of file ---