ADDITIONAL PARAMETERS TO CL

===========================

Additional parameters have been added to CL to increase its

flexibility. In most cases, the usage described in the manual

will be sufficient. The full syntax of CL is:

CL [BAUD,PORT,MODE,STRING]

where you may omit trailing parameters.

The parameters are:

BAUD

Selects the Baud rate. Should be set to one of 300, 600, 1200,

2400, 4800, 9600 with a default of 9600.

PORT

Selects the serial port. Should be set to 1 or 2 to select the

main port or the secondary port respectively with a default of

1 to select the main port. This parameter corresponds to the

n in the DOS command MODE COMn:baud. See your DOS manual for

more details.

MODE

Selects the mode of operation. Should be set to 0, 1 or 2 with a

default of 0. The default of 0 should be used in a direct

connection and either 1 or 2 should be used when connected to a

modem (with the modem in auto answer mode). When MODE is 0, CL

will immediately attempt to link with the Organiser. When MODE is

1, CL will wait until DSR is asserted before it will attempt to

link. When MODE is 2, CL will wait until DCD is asserted before

it will attempt to link.

STRING

The STRING parameter specifies one or more commands to be sent to

the modem before proceeding with the link connection. By default,

no data is sent to the modem. This parameter would be used to

configure a sophisticated modem. Any backslash character (\) in

the string is converted to a <CR>.

Examples:

(1) Use the secondary port in a direct connection:

CL 9600,2

(2) Use an auto-answer modem, waiting for Carrier Detect:

CL 1200,1,2

(3) Configure a Hayes(TM) command set compatible modem, resetting

the modem and then setting it to auto-answer after 3 rings:

CL 1200,1,2,ATZ\ATS0=3\

SINGLE FLOPPY DISC DRIVE UNITS

==============================

On single floppy disk systems, drives A and B refer to the same

drive with the assumption that A and B represent different disks.

If an access is made to one drive when the last access was to the

other, DOS will prompt you to change the floppy disk. If you are

using the PC as an unattended server which is connected to an

auto answer modem, this prompting for a disk change will lock the

server up (as you are not there to press a key).

You can avoid this situation by using the ASSIGN command to

reroute all floppy drive references for drive B to drive A:

ASSIGN B=A

where you should ensure that drive A, and not drive B, was last

accessed (eg by making A the default drive). See your DOS manual

for more details on the ASSIGN command.

ABOUT THIS DISK

===============

Along with the Comms Link software and this file, this disk contains

further text files with the extension '.doc'. These files contain

the same information as this one, in foreign languages.