ZX81TAP v1.01                                         J. Elliott, 19 July 1995.

===============================================================================

ZX81TAP transfers ZX81 tape files into a Spectrum emulator (or a real

Spectrum).

Instructions for use:

You will need an emulator with "true" tape emulation, or a Spectrum with a

way of transferring information to the target ZX81 emulator (eg serial link).

Load ZX81TAP.SNA, and play your tape into it.

The BASIC program in ZX81TAP.SNA can be altered to your own purposes. It

is currently written to behave well with Z80. For your information, the

machine-code section is CODE 60000,256.

ZX81 tapes are very sensitive to load levels; if the tape volume is too high,

the signal will be distorted, while if it is too low, the computer will not

detect it. Normally, a volume level of around three-quarters suffices, but this

varies from tape to tape.

To assist with detection, a "monitor box" is located in the top left-hand

corner of the screen. If a signal is being detected, it flickers; otherwise

it remains steady. When the file has loaded completely, it goes blank. This

will not be visible if you use Z80 in its Real Mode. The ability to hear the

ZX81 tape signal also helps the setting of volume.

Once the file has loaded, the method used to save it depends on your

emulator:

Z80 v2 saves as .TAP files (set this option in the "Tape" menu). Once you

have saved the .TAP file, you have to use a binary editor to remove

the first 23 bytes.

Z80 v3 can save as .TAP files or arbitrary blocks of memory. The advantage

of the latter is that no binary editing is required.

PJPP   saves only as .SNA files. To retrieve the ZX81 file, remove the

first 16411 bytes and then take the next "zx81l" bytes (see the BASIC

program).

Bug reports, comments etc. to:   elliott.teaching@physics.oxford.ac.uk

_______________________________________________________________________________